## Vehicle Maintenance Checklist

Keep up with a preventative maintenance schedule to maximize your vehicle’s lifespan and performance. Use this list to know what – and when – tasks should be done to help keep your car in proper working order.

Service intervals can vary from make and model. Be sure to follow your vehicle’s maintenance schedule, no matter how many miles you drive annually.

### Service Immediately

- Check engine light – This warning appears when the car’s control system discovers a problem. There is no reason to panic when you see this yellow warning light, but don’t delay a visit to the repair shop. Don’t reset a check engine light on your own without knowing why it came on in the first place. A mechanic can run diagnostic tests to determine the cause.

- Headlights/taillights – Check for blown fuses if a light goes out. Replace the light if that isn’t the problem. Driving with burned-out lights is unsafe and can bring you a traffic ticket.

- Tire pressure light – A car’s tire-pressure monitoring system will alert you when the air pressure in a tire drops below a certain amount. Low air pressure can bring unsafe driving conditions, so inflate your tires to proper levels as soon as possible.

### Monthly Check-up

- Fog lights, turn signals, brake, and parking lights – It’s relatively easy to notice a headlight that isn’t working. Others aren’t as obvious, so walk around the car monthly to visually inspect the lights.

- Oil and coolant levels – Check levels when the engine is cool at least once a month and always top off the levels before making a long trip.

- Tire pressure and tread depth – Tires are essential to safe driving. Regularly inspect your tires and the spare for uneven wear, proper air pressure, and adequate tread depth. Use a penny to check the tread depth. If the top of Lincoln’s head is covered, there is still more than 2/32” of depth remaining, meaning there is still adequate tread left.

- Windshield wiper fluid – Be sure the reservoir has an adequate supply of wiper fluid. It’s impossible to drive safely with an obstructed view, something that can result from a dirty windshield.

### 3-Month Check-up

- Oil and filters – Engines that use conventional motor oil can be on a 3-month/3,000-mile interval. Those using synthetic varieties might have up to 5000 - 10,000 miles between oil changes.

- Battery and cables – Make sure the battery and cables have tight connections and no corrosion or leaking fluid.

- Belts and hoses – The serpentine belt and other belts in the engine compartment shouldn’t look glazed, cracked, or frayed. Hoses shouldn’t leak or have cracks or bulges.

- Power steering fluid – Check the power steering fluid level when the engine is warm and add more when needed.

- Wiper blades – Driving with worn wiper blades is a safety hazard because of reduced visibility when it rains. Inspect the blades seasonally and replace them if they’re damaged or no longer clear the windshield.

### 6-Month Check-up

- Rotate tires – Rotating tires helps extend their life by balancing the tread wear and can help prevent noise and vibration problems. Check the owner’s manual beforehand because some types of tires and wheels shouldn’t be rotated or must be rotated in a particular way.

- Wax vehicle – Wash your car regularly and apply a wax coating at least twice a year to help protect your car’s finish from rust.

- Exhaust system – Look for and repair any damage, especially if the muffler is making noise.

- Battery performance check – Your car won’t start without a good battery. Beginning when the battery is three years old, test it twice a year at your auto parts store.

- Chassis lubrication – Your owner’s manual will say if the chassis, steering, and suspension systems require periodic lubrication.

RELATED ARTICLE: How to Jump-Start an Electric Car Battery

### 1-Year Check-up

- Air filters – Cabin air filters help clean the air inside the car and require annual replacement. However, engine air filters keep debris out of your engine and should be inspected during oil changes.

- Brakes – Inspect the brake system, including the brake fluid, brake linings, rotors, and brake pads, to help ensure the proper operation of these critical components. The lifespan of brake pads largely depends on the operator’s driving style.

- Inspect shocks and struts – Take your car to the shop if you notice a decrease in smoothness when driving. Shocks and struts are an essential part of the car’s steering system and should be inspected by a professional.

- Coolant/antifreeze – Replace every year. Flush the coolant and the entire cooling system after 60,000 miles.

### 2-Year Check-up

- Ignition system – Good quality spark plugs, plug wires, coils, and other electrical components can last up to 100,000 miles. Even so, checking spark plugs starting at 30,000 miles is a good idea. Rough running or hard starting can indicate that they’re beginning to fail.

- Transmission fluid – Check transmission fluid levels regularly and add more when needed. You can expect to change transmission fluid between 30,000 and 60,000 miles in a manual transmission vehicle and between 30,000 and 100,000 miles in an automatic transmission one.

- Fuel filter – Manufacturer guidelines for fuel filter replacement vary. Some suggest replacement at 30,000 miles.

### Long-Term Check-up

- Transfer case fluid –The transfer case shifts power from the transmission to the axles in a 4-wheel-drive or all-wheel-drive vehicle. Have a professional check transfer case fluid according to manufacturer recommendations.

- Front and rear differential – Differentials are devices that split the torque from the engine and send power to the tires to propel the car. The differentials require lubrication, and a professional should check them according to manufacturer recommendations.

- Change tires – Tires can last from six years to 10 years. Check often for adequate tread depth greater than 2/32 of an inch.

- Battery – Test the battery beginning at three years. It’s time to replace the battery after five years.

- Timing belt – Replace following the owner’s manual guidance, typically between 60,000 and 90,000 miles. Not all vehicles have timing belts. Yours might have a timing chain, which often needs no periodic maintenance (or replacement) unless there’s an issue.

These milestones for car maintenance are general guidelines and not an exhaustive list. Carefully follow your car manufacturer’s recommendations on scheduled vehicle service and use qualified mechanics to perform work on your car.